Different Types of Flood defence

Flood defences can be built in an area before or during high water level, to protect the surrounding land from being inundated (flooded). Areas of rivers which flood regularly have defences built as a result of earlier floods.

Long term defences like these aim to protect from regular flooding. They include:

- Artificial levees. When rivers overflow onto the floodplain, silt and sediment settle out of the water and are left on the floodplain.  When the river level returns to normal, the floodplain at either side of the river channel has been raised. This process repeats itself over time to create raised riverbanks, or levees:

Natural levee formation

Levees do not form naturally on all rivers because only a few rivers carry enough sediment. Therefore artificial levees are needed: they are made from soil and grass. They form a narrow raised ridge of land along the banks of the river. The presence of artificial levees means that the river water can rise higher without flooding surrounding land.

Levees are very effective at protecting nearby land from small floods, but are rarely large enough to protect from all sizes of floods.

Protected side of the levee

Sea-side of the levee

Cycle path along top of levee

An artificial levee, Holland.

- River alteration. This involves widening or deepening (by digging a deeper channel) the channel so that more water can be held.

‘Relief channels’ may also be dug to allow extra water to be diverted away from the main river channel. Examples on pg 16-17 of Water, Landforms, and People.

- Dredging of the river bed. This involves scraping away sediment, mud, sand and rubbish which has built up over time on the river bed. This makes the river channel deeper, meaning that more water can flow through before the channel overflows:

River channel before Dredging

Sediment, mud, rubbish, etc before dredging

Water depth at high water level

River channel after Dredging

Water depth at high water level.

Short term defences are usually built when a flood is expected, or when other flood defences such as levees have been damaged by the flood. They include:

- Strengthening the edge of the floodplain and levees with concrete blocks:

Blocks being filled with concrete to stop floodwater from spreading.

- Digging channels and laying pipes to allow flood water to drain away from waterlogged areas:

Pipes being used to remove water from waterlogged farmland.

- Pumping water away from flooded areas, e.g. residential areas.

- Using sandbags to stop or delay water entering houses and businesses.

To do in exercise books:

- What is the job of flood defences?

- When do short term flood defences tend to be built?

- Explain in your own words how natural levees are formed.

- Why are artificial levees needed?

- How does river channel widening prevent flooding?

- How does river channel deepening prevent flooding?

- How do relief channels prevent flooding?