River in a Tray

Note: It is a good idea to test this activity out prior to using it with the class as it requires some experimentation before use.

Instructions:

1. Place the plastic tray on a bench/table and then put two house bricks under one end to create a slope down to the open end. The plastic bucket should be placed at the open end so that it can catch any sand and water. Fill the tray with sand.

2. The aim of the activity is to create the pattern of a river flowing across the sandy landscape. You need to create raised hill/mountainous areas round the top of the tray using stones/rocks.

3. At the raised end (top) of the tray pour about 2 litres of water from the watering can using the rose; it should be poured very slowly. The water should flow down the slope and start to create a channel.

4. The children should note on their worksheet (3.2) the shape of the channel, if the river makes any bends and how the water flows differently in the hills and mountains compared to the flatter areas. They should also draw a labelled diagram of the river.

5. It is possible to demonstrate different situations found on a river;

- place stones across the river to create a dam;

- pour the water faster so that it creates flood conditions;

- make a waterfall by placing stones across the whole tray;

- vary the shape, size and positioning of the mountains.